FILE:			addend.doc

VERSION:		Release 1 SpringBoard Interface Software 14/08/87

SpringBoard Supervisor version 1.30

Interface Software version 0.4k

DESCRIPTION: 	This file describes alterations and additions to the System

Guide.

See also the file examples.doc in the examples directory.

===========================================================================

CONTENTS

ADDED FEATURES

New commands

DELETE

MODE

MONITOR

DOS_NAMES,  ADFS_NAMES

FS_ERRORS

EXEC

New ARMINT calls

ARMREENT

ARMEVENT

ARMERROR

ARMADDR

Machine Identification

MANUAL CORRECTIONS

Clarifications

Commands

ARMINT calls

KNOWN BUGS

APPLICATIONS PROBLEMS

===========================================================================

ADDED FEATURES

New Commands

DELETE

Works as the MSDOS command.

MODE

Allows selection of IBM screen mode used when connected.  Any mode is allowed

but the VDU driver assumes the screen is 80 columns by 25.  See the section

on screen usage in this addendum.

MONITOR

This is a debugging aid and allows text to be produced to indicate the state

of the SpringBoard.

Turning ARMINT debugging information on alows one to investigate which ARMINT

calls have been issued to the SpringBoard along with the rest of the

registers.

Turning R2 debugging on will display all tube register two requests that are

parsed by the system.

The display is of the form:

r2_parser( <tube request> )

followed by actions done on that request.

NOTE: The ARMINT call ARMEXEC does some of its own parsing of the tube and

therefore not all of the debugging information is seen.

DOS_NAMES,  ADFS_NAMES

This allows switching between ADFS and DOS file nameing conventions.  The

most obvious difference is that adfs uses period as a directory seperator

not backslash.

FS_ERRORS

The filing system calls such as OSFILE default to generating errors if

there is a filing system error, for example if an attempt was made to

load a non-existant file.  In some cases this may not be required and instead

one may simply wish to investigate the return codes to determine whether

or not an error has occurred.  For this reason the error messages can be

suppressed by typing FS_ERRORS OFF.

EXEC

This command allows input to be taken from a text file as if it had been

typed at the keyboard.  Exec files may not be nested.  If a file has a .AXM

extension then just typing the filename is sufficient,  the EXEC is not

needed.  This is similar in idea to DOS batch files.

New ARMINT calls

ARMREENT

This call enables one to check that no-one else is currently running the

interface software.  This call only of all the ARMINT calls is reentrant.

It returns 0 in ax if it is OK to make other ARMINT calls and 1 otherwise.

ARMEVENT

This call generates an event on the SpringBoard.

Enter with dx indicating the event number and bx, cx indicating the event

specific information.  See the ARM utilities manual to see how to catch

the event interupt on the ARM.

ARMERROR

This call generates an error on the SpringBoard.  It passes an error message

and error number to the ARM interupting it.

Enter with ds:dx ponting to the null terminated error text and with the error

number in bx.

ARMADDR

This call alters the interface software's interpretation of the base address

for the current SpringBoard.  Setting this base to an illegal address will cause

unpredictable results.

Enter with dx indicating the base of the new SpringBoard in the range 300 (hex)

to 3C0 (hex) in increments of 8.

The interface software has only one set of state information.  Ie. current

directorys,  tube state etc.  If the two cards are not in the same state

then switching between cards is not advisable.  The best occasion to change

is when ARMPOLL reports there are no commands pending.

Machine identification

The Tube protocols used by SpringBoard are derived from those used on the

BBC B 2nd processor interface. There are a common set of o/s facilities

provided across the whole range of Acorn machines. In order to identify the

exact environment that a program is running in there are a number of o/s

calls to establish the machine, filing system, processor etc. These calls

are,

Osbyte R0=0,R1=0 which prints up an o/s identity string on the

screen.

Osbyte R0=0,R1=1 which returns an o/s identity number in R1, in the

case of SpringBoard this is 7.

Osbyte R0=129, R1=0, R2=255 which returns a machine type, which in

the case of SpringBoard returns in R1=0xAF

The OSARGS option on OSFILE with R0=0, R2=0 returns in R0 the filing

system number which for SpringBoard is 8, corresponding to the Acorn

Advanced Disk Filing System (ADFS).

Manual Corrections

Clarifications

Screen usage

When connecting to the ARM the current DOS screen is saved and the ARM's

screen is restored.  This screen save and restore works whenever the current

DOS screen is the monochrome adapter or a CGA screen mode.  The screen mode

used for the ARM is mode 7 or if this is not available then the mode the

user was in at initialisation is used.  Screen save and restore will not

work if only an EGA adaptor is fitted.

Interface initialisation

Dos applications which attempt to make ARMINT calls before the interface

software has been installed will fail unpredictably, usually with the

'Divide overflow' message from Dos. This is because the interrupt vector has

not been initialised. Applications should check for the presence of the

interface software before attempting to make ARMINT calls. This can be done

by checking the signature placed in the next interrupt vector. The actual

value is contained in the 'armintc.h' file with the example programs.

Screen driver

When an ARM application writes to the screen (without using STDIOON)

character codes below 32 have special interpretations (for example cursor

positioning, text windows etc) These codes are derived from the Acorn Master

128 operating system VDU driver and desribed in the 'Master reference manual

part 1'. Those calls relating to graphics manipulation are not implemented

for the SpringBoard. These codes are recognised and ignored. If for instance

a PLOT command, character code 0x19, is sent then the PLOT byte and the next

five parameter bytes sent to the screen are discarded. This can lead to

confusion when such a code is accidently input from the keyboard as the next

few keypresses will be discarded depending on the exact code.

Of these VDU codes the following are ignored, 0x11-0x14, 0x16-0x19 and 0x1d.

ARM -X

When connected by the -x option of the front end the only way to disconnect

is when an ARM application issues a SWI EXIT.  Inbuilt commands (eg. DIR

MEMORY) do not generate a SWI EXIT.  This can be generated at the

supervisor command line by the command QUIT.  The disconnect methods EXIT

and <ctrl esc> are disabled.  The intended use of the -x option is to

make the existance of the ARM card transparent to the PC user,  allowing

turnkey ARM applications.

Current directories

The SpringBoard has its own current directory.  This is not related in any

way to the DOS current directory.  The Springboard current directory also

includes the drive,  unlike DOS which has a current directory for each drive.

This is to allow switching between concurrent applications in the PC and

ARM environments. This may cause confusion on connect and disconnect between

the two environments.

Floating point emulator

After loading the FPE the ARM does not recognise ARM supervisor commands.

This is because the emulator provides its own command line replacing the

supervisor command line. Access to the supervisor is possible with the use

of the GOS command, with QUIT then returning you to the FPE.

Cache command

The example in the manual is incorrect, it should read,

CACHE TWIN.AXH TW

where you are in the same directory as TWIN.AXH. Alternatively a full

pathname can be entered.

Scrolling

In the SpringBoard environment there is a page mode for screen output

available, this can be switched on by <ctrl-N> and off by <ctrl-O>. When

page mode is on output will pause automatically after every three-quarters

of a screen. Output is restarted in page mode after a pause by pressing the

SHIFT key. Alternatively output can be stopped by <ctrl-NUMLOCK> or

equivalent. The <ctrl-S>, <ctrl-Q> pausing mechanism is not available in the

SpringBoard environment see the notes on the VDU driver regarding what

the control character codes do.  Escape does not work while the screen

output is paused.

Assembler programs

At the end of an assembler program, flow of control must be passed back to

the SpringBoard executive. The final instruction must be a SWI EXIT.

Manual Errors

ARMBKW

The reference in 6.2.8 ARMBKW to an error return code for BKBADOP of &15 is

incorrect and should read &16.

ARMBKGET/ARMBKPUT

The references in sections 6.2.15 and 6.2.16 to an address in SpringBoard

memory are incorrect. The correct registers are,

ES - bits 16-31

SI - bits 0-15

not ES,BP as in the manual.

KNOWN BUGS

Parsing of filenames

A tree path containing ..\.. is not correctly parsed.  The sequence is

ignored.

Paged mode

When paused waiting for the shift key to be pressed the escape key is not

immediately reacted to.

APPLICATIONS PROBLEMS

DEBUG

The manual fails to mention that in order to use DEBUG on a file the option

-dbug must be given to link.

M2RUN,  DECOBJ, MERGECF

All filenames passed to it on the command line must have \\ instead of \.